ANNEX  B

| COVER SHEET & Legend Archival and other Non Current Records: Types of Facilities & Services Recommended |  |  |  |  | Date: 27 May 1993 Ref: IRM/RAM/93-052/R02 Sub: AD-251 /Archive |
| --- | --- | --- | --- | --- | --- |
| Para | Weight | Result | Facility or Service Description | ARCHIVE BUILDING [AVRSA) [High-security storage] | Non Current Record Storage (NCRSA)[less secure space] |
|  |  |  |  | R, P, m, n * |  |
|  |  | * | Results Legend | Type of Record Stored: | Type of Record Stored: |

|  |  |  | 5 |  |  | = Yes, meets the Standard |  |  | Appraised: - ARCHIVAL, Historic or - VITAL and |  |  |  | Appraised: - LIMITED Value (Short Term = less than 10 Years) and |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 3 | = Partial, meets part of standard or for some of area |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 1p | = No, but plan to meet |  |  |  |  |  |  |  |  |
|  |  |  | 1f |  |  | = No, due to financial constraints |  |  | Non appraised: but Probable Archival [from Substantive Activities or Evidential records] |  |  |  | Non appraised: but Probable Limited Value [from facilitative Activities or Routine, Background and temporary Records] |  |  |  |
|  |  |  | 1c |  |  | = No, due to non-financial (other) constraints |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 0 |  |  | = Does not meet standard |  |  | * Standard Legend |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | R = Required Standard (Recommended) P = Preferred: Higher standard recommended but not required m = Minimum Standard is acceptable for this area n = Not required or applicable |  |  |  |  |  |  |  |
|  |  | # |  | Weight = importance [scale 10 = most important] |  |  |  |  |  |  |  |  |  |  |  |  |
| A |  | Para = Paragraph Number [See Page 2 for details and subjects covered] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: |  | For definition of record or appraisal terms see UNICEF Policy and Procedure Manual, Book J: Administration. [CF/MN/P.J], Chapter 10; special attention to Section 1, appendix C and Section 4, appendix C. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | See IRM/RAM/93-055 for reference works to be consulted for evaluation of Archives facility or Non-current Record Storage area [and previous Unicef responses on related subjects] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| TABLE of CONTENTS Archival and other Non Current Records: Types of Facilities & Services Recommended |  |  |  |  |  |  |  |  |  |  |  |  | Date: 27 May 1993 Ref: IRM/RAM/93-052/R02 Sub: AD-251 /Archive |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Para | Weight | Result |  | Facility or Service Description |  |  | Details |  |  | Subjects Covered |  |  |  |  |  |
| A |  |  |  | SECURITY |  |  | page 3 |  |  | Keys, electronic monitoring, researchers, neighborhood, procedures, fire protection |  |  |  |  |  |
| B |  |  |  | ENVIRONMENTAL Concerns |  |  | page 6 |  |  | Air temp, humidity, dust, testing, lighting, hazardous building material, |  |  |  |  |  |
| C |  |  |  | RESEARCH related items |  |  | page 8 |  |  | Assistance available, related records, indexes, databases |  |  |  |  |  |
| D |  |  |  | LOCATION of Facility |  |  | page 9 |  |  | Distance, access to public facilities/ services for staff, loading dock, |  |  |  |  |  |
| E |  |  |  | SPACE Layout, Usage and COST |  |  | page 10 |  |  | Space available, cost, specialized areas, usable space, floor weight limit |  |  |  |  |  |
| F |  |  |  | EQUIPMENT |  |  | page 13 |  |  | This section should be used as noted below: |  |  |  |  |  |
|  |  |  |  | EQUIP - (Site determined) |  |  |  |  |  | When considering New space: Public phones, and equipment left by previous tenant. |  |  |  |  |  |
|  |  |  |  | EQUIP - (Evaluation & Use) |  |  |  |  |  | When evaluating space now used or equipment required for good operations and planning for improvements. Computers, phones, microfiche readers, photocopiers, carts, fork lift, tables, desks, shelving, boxes. |  |  |  |  |  |
| G |  |  |  | ADMINISTRATION & Staff items |  |  | page 16 |  |  | For evaluating administration of facility and planing future changes: Staffing, training, cleaning & garbage, supplies |  |  |  |  |  |
| H |  |  |  | LEGAL, Tests and Certification |  |  | page 18 |  |  | Certificate of occupancy, tests [air quality, fire protection, hazardous building material - see also para I, Physical Plant and J, Commercial Facilities] |  |  |  |  |  |
| I |  |  |  | Physical Plant |  |  | page 18 |  |  | Wiring, electrical power, heating, cooling, elevator tests |  |  |  |  |  |
| J |  |  |  | Commercial Facilities (possible use for temporary records and emergencies) |  |  | page 20 |  |  | Concerns when commercial facilities are considered for use due to emergency or non availability of suitable UN controlled storage areas. |  |  |  |  |  |
| Archival and Non Current Records: Facilities & Services |  |  |  |  |  |  |  |  |  |  |  |  | Ref: IRM/RAM/93-052/R01 |  |  |
| Para | Weight | Result |  | Facility or Service Description |  |  |  | ARCHIVE, Vital Record Store [AVRSA) [High-security] |  |  |  |  | Non Current Record Storage Area (NCRSA) [less secure space] |  |  |
| A |  |  |  | SECURITY |  |  |  | Note: If both facilities [AVRSA & NCRSA] are on same site and access can be obtained through either entrance the higher level of Security must be used for both. |  |  |  |  | If an effective security check between AVRSA and NCRSA exists. Both may be treated as separate facilities for security standards |  |  |
| A1 |  |  |  |  |  | Security Level 1 (Dead Bolts, double locks on all Doors accessible from adjoining Spaces, Street or Fire Escapes) |  |  | R |  |  | R |  |  |  |
| A1a |  |  |  |  |  | Bared windows on Lower Levels |  |  | R |  |  | R |  |  |  |
| A1a1 |  |  |  |  |  | Bars on widows accessible to Fire Escapes |  |  | R |  |  | R |  |  |  |
| A1a2 |  |  |  |  |  | Bars on widows accessible from Roof |  |  | R |  |  | P |  |  |  |
| A1b |  |  |  |  |  | Procedure for Keys Access all areas and times - Weekend, Overtime, special retrieval or emergency. Identifies who is authorized to enter or accompany in emergency. |  |  | R |  |  | R |  |  |  |
| A1b1 |  |  |  |  |  | Keys for Access - Key Box with all keys marked and accounted for. |  |  | R |  |  | R |  |  |  |
| A1b2 |  |  |  |  |  | Keys for Access - Key box or elsewhere notes who has all copies. Log of any exceptions where key is given to someone not on list. |  |  | R |  |  | P |  |  |  |
| A2 |  |  |  |  |  | Security Level 2 (Alarmed doors and Windows) |  |  | R |  |  | P |  |  |  |
| A3 |  |  |  |  |  | Security Level 3 (Electronic Monitoring some hours: on or off site) |  |  | R |  |  | m |  |  |  |
| A4 |  |  |  |  |  | Electronic Monitoring off site |  |  | m |  |  | m |  |  |  |
| A4a |  |  |  |  |  | Electronic Monitoring and random 24 hour checking of locked doors by security personnel |  |  | m |  |  | m |  |  |  |
| A5 |  |  |  |  |  | Above level of inhibitors to Entrance [Double locked, alarmed] + "Card Key" record of who enters |  |  | P |  |  | m |  |  |  |
| A6 |  |  |  |  |  | Identification of Researchers and sign in |  |  | R |  |  | m - Only box retrieved. Requesting office notes researchers. |  |  |  |
| A6a |  |  |  |  |  | Manual Searching of what is brought into Facility (arms, cutting implements, big bags or cases) |  |  | R |  |  | n |  |  |  |
| A6a1 |  |  |  |  |  | Metal detector for visitors and researchers |  |  | P |  |  | n |  |  |  |
| A6a2 |  |  |  |  |  | Researchers sign Out |  |  | R |  |  | n |  |  |  |
| A6a3 |  |  |  |  |  | Manual Searching of what is brought out of Facility (big bags or cases) |  |  | R |  |  | m |  |  |  |
| A6b |  |  |  |  |  | Recording of types of Research being performed |  |  | R |  |  | m - by requesting office |  |  |  |
| A6c |  |  |  |  |  | Periodic checking on use of records |  |  | R |  |  | m - by requesting office |  |  |  |
| A6c1 |  |  |  |  |  | Automatic TV (taping and/or monitoring) of Researchers work/reading room. All informed of Policy |  |  | R |  |  | m - by requesting office |  |  |  |
| A6d |  |  |  |  |  | Protection of personnel Belongings of Researchers - Locked Storage or Security person with tag check |  |  | R |  |  | m (for staff working at site) |  |  |  |
| A7a |  |  |  |  |  | Fire protection - Alarms on Site (detection) |  |  | R |  |  | R |  |  |  |
| A7b |  |  |  |  |  | Fire Protection - Alarms notification to alternate site (detection). |  |  | R |  |  | P |  |  |  |
| A7c |  |  |  |  |  | Fire Protection - procedures and equipment for Manual suppression (Water Hose, Extinguisher [water or foam] and sand pails; |  |  | R Note water has potential paper damage). |  |  | m |  |  |  |
| A7d |  |  |  |  |  | Fire Protection - procedures and equipment for automatic suppression (water or Gas). |  |  | R Note gas has potential people problems, Water has potential paper damage. |  |  | m |  |  |  |
| A7da |  |  |  |  |  | Fire Protection - procedures and equipment for automatic suppression (water or Gas) in specialized separate areas. Computer control rooms or storage areas for electronic and high security [evidential records] or unattended locked areas. |  |  | R Note gas has potential people problems, Water has potential paper damage. |  |  | m |  |  |  |
| A8 |  |  | 10 |  |  | Confidence of users in Security arrangements (Confidence of Public and Managers that records protected under accepted and agreed standards) |  |  | R |  |  | P |  |  |  |
| A8a |  |  |  |  |  | Inviolability of Records (able to attest that records were not out of direct control during lifecycle) [check A/CN.4/438 7 May 91 for wording - Ownership.] |  |  | R |  |  | R [See also notes on Commercial Storage, para J when records may be considered for temporary storage or in an emergency] |  |  |  |
| A8b |  |  |  | Records physically separated (from various other non-record items in storage. Restricted access) |  |  |  | R |  |  |  |  | R |  |  |
| A8c |  |  |  | Entrances/Neighborhood Safe for staff |  |  |  | R |  |  |  |  | R |  |  |
| A8c1 |  |  |  | Immediate Neighborhood Safe from possible manmade or natural disaster [e.g. flooding, armed conflict, highly political organizations, storage or production of explosives and processes requiring chemicals] |  |  |  | R |  |  |  |  | R |  |  |
|  |  |  |  |  |  | Risk and Vulnerability: Security analysis completed to evaluated the risk/vulnerability of staff from placement of UN related records in facility. |  |  | R |  |  | R |  |  |  |
| A8c3 |  |  |  |  |  | Staff preparation: All staff given instruction regarding procedures to be followed in event of emergency. Which records to attempt to save or abandon. Phone numbers current, Organizational authority clear. |  |  | R |  |  | R |  |  |  |
| B |  |  |  |  |  | ENVIRONMENTAL Concerns |  |  |  |  |  |  |  |  |  |
| B1 |  |  |  |  |  | Air Quality - Dust etc. |  |  | R |  |  | m |  |  |  |
| B1a |  |  |  |  |  | Regular Testing for Air Quality, Dust, Acid, Bacteria. Records Kept. |  |  | R |  |  | m |  |  |  |
| B1a1 |  |  |  |  |  | Specific Standards for Air Quality, Dust, Acid, Bacteria to be added for each medium. |  |  | R |  |  | m |  |  |  |
| B1b |  |  |  |  |  | Testing for Humidity - Off hours and all areas. Records kept. |  |  | R |  |  | m |  |  |  |
| B1c |  |  |  |  |  | Records requiring special Temp/Humidity kept in Quality controlled area. |  |  | R [for each medium of storage see Para B.2 below] |  |  | m |  |  |  |
| B2 |  |  |  | Air Temperature/Relative Humidity (see details for media below; Source = ICA Handbook Series Volume 6, 1988 (pp104-113)] |  |  |  | Temp. |  |  | Relative Humidity |  | Note for Relative Humidity: Low = Dryness [Paper becomes brittle and may crack or disintegrate] High = Moisture [Microbes & Mold can grow - obscure text] |  |  |
| B2a |  |  |  |  |  | T/H for Paper and Parchment |  |  | 13-18 C 56-65 F |  |  | 55-65% |  | m (extreme can damage paper within ten years) |  |
| B2b |  |  |  |  |  | T/H for Black & White Photographic material (including microfilm) |  |  | 11-13 C 52-55 F |  |  | 30-40% |  | m (extreme conditions could damage photo within ten years) |  |
| B2c |  |  |  |  |  | T/H for Colored Photographic Material |  |  | 4-6 C 39-43 F |  |  | 30-40% |  | n (NCRSA not suitable) |  |
| B2d |  |  |  |  |  | T/H for Colored Films |  |  | -5 C 23 F |  |  | 30-50% |  | n (NCRSA not suitable) |  |
| B2e |  |  |  |  |  | T/H for Magnetic Medium (including Video tapes, sound recordings, computer tapes and disks) |  |  | 17-19 C 63-66 F |  |  | 35-45% + Dust Filter |  | n (NCRSA not suitable) |  |
| B2f |  |  |  |  |  | T/H for Staff & Public |  |  | 17-22 C 65-75 F |  |  | 40-70% |  | m (staff use only & for limited periods in extremes of hot or cold; portable heaters and fans must be available) |  |
| B3 |  |  |  | No Hazardous Building Material (beware of floors above and below records: asbestos, chemical, fumes and/or leaks, drainage and sewage) |  |  |  | R See also Test Legal PARA H |  |  |  |  | R |  |  |
| B3a |  |  |  |  |  | Air condition and Heating Plant separate from all Record Storage Areas, ventilated and drained away from records |  |  | R |  |  | R |  |  |  |
| B3b |  |  |  |  |  | Production or Preservation process which requires chemicals [film developing, book binding] is separate from all Record Storage Areas; ventilated and drained away from records |  |  | R |  |  | R |  |  |  |
| B4 |  |  |  |  |  | Office and work area LIGHTING (accessioning, sorting, record keeping) |  |  | R |  |  | m [need at least one room to perform some record keeping tasks] |  |  |  |
| B4a |  |  |  |  |  | Research Area Lighting (task lighting - fine print) |  |  | R |  |  | n |  |  |  |
| B4b |  |  |  |  |  | Stacks Storage Area Lighting (lights down aisles not across) (Light switches clearly labeled) |  |  | R |  |  | R |  |  |  |
| B4c |  |  |  |  |  | Direct Sunlight minimized for all areas where Potential Long Term records will be sorted, processed or reviewed. |  |  | R - solar can cause damage to records |  |  | R |  |  |  |
| C |  |  |  |  |  | RESEARCH related items |  |  |  |  |  |  |  |  |  |
| C1 |  |  |  |  |  | Assistance available to researchers (Staff to direct inquiries and provide records) [see also Para F, Equipment & Para G, Administration/Staff] |  |  | R |  |  | m (provided by original office or others at main site) |  |  |  |
| C1a |  |  |  |  |  | Produce records under Supervision - Security Concerns (see also Para A) |  |  | R |  |  | m (provided by requesting/original office) |  |  |  |
| C2 |  |  |  |  |  | Access to related records of other departments or agencies. Sufficient practical cooperation arrangements for description, access and retrieval. |  |  | R |  |  | n |  |  |  |
| C2a |  |  |  |  |  | Ease of Access to records for officials. [See also para. D Location] |  |  | R |  |  | n |  |  |  |
| C2b |  |  |  |  |  | Integrated with related events via updated indexes, databases and other finding aids. |  |  | R |  |  | n |  |  |  |
| C2c |  |  |  |  |  | Predecessor agencies and offices records easily available |  |  | R |  |  | n |  |  |  |
| D |  |  |  |  |  | LOCATION of Facility |  |  |  |  |  |  |  |  |  |
| D1 |  |  |  |  |  | Distance from headquarters [Preference for evidential and other often referenced records to be on Site] |  |  | R [Within 20 min of Hqs. but prefer within 5 min.] |  |  | P/m (should be within 1/2 hour for emergency requests) |  |  |  |
| D2 |  |  |  |  |  | Distance from other UN Offices in same City with related records |  |  | R [Within 20 min of Hqs. but prefer within 5 min |  |  | m |  |  |  |
| D3 |  |  |  |  |  | Access to Public Eating Facilities |  |  | R |  |  | R |  |  |  |
| D4 |  |  |  |  |  | Access to Public Transport |  |  | R |  |  | R |  |  |  |
| D5 |  |  |  |  |  | Access to Medical/Dental |  |  | R |  |  | m |  |  |  |
| D6 |  |  |  |  |  | Access to Child Care Facility in Area. Should be within ten minutes for mother with children under 2. |  |  | P - Check City or State Gov't list of Accredited facility near location. |  |  | m (only if staff who need CC was assigned for special project lasting more than one day) |  |  |  |
| D7 |  |  |  |  |  | Access to other Public Services (Banks, Library) |  |  | R |  |  | n |  |  |  |
| D8 |  |  |  |  |  | Loading Dock (Truck Back in) |  |  | P (if not exist will significantly increase labor cost/time for accessions and disposals) |  |  | R |  |  |  |
| D8a |  |  |  |  |  | Large Garbage Truck Access (disposal of 600 cartons at a time) |  |  | R (lack will increase labor cost/time) |  |  | R |  |  |  |
| E |  |  |  |  |  | SPACE Layout, Usage and Cost |  |  |  |  |  |  |  |  |  |
| E1 |  |  |  |  |  | SPACE Available/COST |  |  | Evaluate space available compared to requirements |  |  |  |  |  |  |
| E1a |  |  |  |  |  | Total Floor Space Available |  |  | R |  |  | R |  |  |  |
| E1a1 |  |  |  |  |  | % of Floor space "usable" |  |  | (See para E7) |  |  |  |  |  |  |
| E1a2 |  |  |  |  |  | Cost per Square foot at present |  |  | R |  |  | R |  |  |  |
| E1a3 |  |  |  |  |  | Total Cost for Space Available |  |  |  |  |  | R (consider "usable" space) |  |  |  |
| E1b |  |  |  |  |  | Total Floor Space available for future expansion |  |  | R |  |  | R |  |  |  |
| E1b1 |  |  |  |  |  | Cost per Square foot for future |  |  | R |  |  | R |  |  |  |
| E1b2 |  |  |  |  |  | Total Cost for Future Expansion |  |  |  |  |  |  |  |  |  |
| E2 |  |  |  |  |  | Space REQUIRED Now/Future |  |  |  |  |  |  |  |  |  |
| E2a |  |  |  |  |  | Space for all Records now in Storage |  |  | R |  |  | R |  |  |  |
| E2b |  |  |  |  |  | Space for Records storage growth (3 years) |  |  | R |  |  | R |  |  |  |
| E2c |  |  |  |  |  | Space for records storage growth (3yr to 7yr) |  |  | R |  |  | m |  |  |  |
| E3 |  |  |  |  |  | Records in Process IN/OUT |  |  |  |  |  |  |  |  |  |
| E3a |  |  |  |  |  | Space for Records being retrieved for users |  |  | R |  |  | m |  |  |  |
| E3c |  |  |  |  |  | Space for Records coming into Facility; work area to initially store records before accessioned and placed on Shelves or for records being returned after re-use (big shipments can be 100-150 boxes, [5-10 skids]) |  |  | R (Archives may receive multiple shipments before one can be processed assigned location label etc. and placed on the shelves) |  |  | R (Location for each box should be assigned and label printed and placed on transfer Box before it is accepted from original office and sent to storage area) |  |  |  |
| E3d |  |  |  |  |  | Space for Skids of Records to be disposed (need to accumulate 500-600 boxes [20 skids]) |  |  | R |  |  | R |  |  |  |
| E3d1 |  |  |  |  |  | Space for a shredder and Access, work space, (Note residue - grows after shredding must be stored until garbage pickup.) |  |  | ? |  |  | ? |  |  |  |
| E4 |  |  |  |  |  | Office Areas: Space for all staff assigned or expected growth for terms of lease |  |  | R- Based on recomended & preferred standard for size {square footage} of workspace and access to Windows etc. |  |  | m |  |  |  |
| E4a |  |  |  |  |  | Office Area (Record Receiving, preservation etc) Research Area covered below |  |  | R |  |  | m |  |  |  |
| E4b |  |  |  |  |  | Office Work area for sorting and checking records to be accessioned or provide finding aids (space to view contents of many boxes simultaneously) |  |  | R |  |  | m |  |  |  |
| E4c |  |  |  |  |  | Office Areas for Administration (and staff doing preliminary research for retrieval or updating finding aids) |  |  | R |  |  | m |  |  |  |
| E4d |  |  |  |  |  | Office Areas for Security staff/equipment and reception |  |  | R |  |  | m |  |  |  |
| E5 |  |  |  |  |  | Lunch Room Separate from Records Areas |  |  | R |  |  |  |  |  |  |
| E5a |  |  |  |  |  | Reliable drinking Water and Refrigerator on site |  |  | R |  |  |  |  |  |  |
| E5b |  |  |  |  |  | Vending Machine or Cafeteria within 5 min. |  |  | R |  |  | m (access to public facility in area) |  |  |  |
| E6 |  |  |  |  |  | Storage Space for records packing and marking supplies and related equipment |  |  | R |  |  | m |  |  |  |
| E7 |  |  |  |  |  | Floor Layout Efficency for Workflow and Usage or Weight Capacity |  |  | R |  |  | R |  |  |  |
| E7a |  |  |  |  |  | Floor Usage Capacity (function of ceiling height, weight load supportable and % of floor space usable. |  |  | R |  |  | R |  |  |  |
| E7c |  |  |  |  |  | Floor Layout for Workflow (efficency, ease of use, control of security and appearance). |  |  | R |  |  | R |  |  |  |
| E7c |  |  |  |  |  | Layout Workflow (account for closeness of related functions from receit of shipments through accessions permanent retentions or disposalarea). |  |  | R |  |  | R |  |  |  |
| E7c |  |  |  |  |  | Layout for Workflow (account for reception and security control of researchers - entering stacks unattended, rest rooms close to Researchers and can not take documents out of security area). |  |  | R- Staff who work most directly/often with researchers should have close proximity: Reception, Security, Research Ass't. |  |  | m- Researchers only enter on an exceptional basis and do not enter unaccompanied |  |  |  |
| E7a1 |  |  |  |  |  | % of floor space usable must account for common services: elevators, stairs, main aisles, toilets, wiring closets, heating and cooling equipment and sufficient access for repairs. |  |  | R |  |  | R |  |  |  |
| E7b1 |  |  |  |  |  | Floor Weight Capacity 1 (ability to support standard shelving units [6 levels ht], fully loaded with 3 foot aisles) |  |  | R |  |  | R |  |  |  |
| E7b2 |  |  |  |  |  | Floor Weight Capacity 2 (ability to support standard shelving units [8-10 levels ht], fully loaded with 3 foot aisles) |  |  | ? (Drawback: Higher shelves Require Stepladders for placement of boxes = more staff time for access and retrieval etc., greater possibility of accident) |  |  | ? |  |  |  |
| E7b3 |  |  |  |  |  | Floor Weight Capacity 3 (Ability to support compact shelving units [6 levels ht], fully loaded with one movable aisle for each 3-5 rows of shelves) |  |  | P |  |  | P |  |  |  |
| E7b4 |  |  |  |  |  | Floor Weight Capacity 4 (Ability to support compact shelving units [8-10 levels ht], fully loaded with one movable aisle for each 3-5 rows of shelves) |  |  | ? (Drawback: Higher shelves Require Stepladders for placement of boxes = more staff time for access and retrieval etc.) |  |  |  |  |  |  |
| E8 |  |  |  |  |  | Layout - Rows/Aisles (sufficient Spaces for Quick retrieval |  |  | R (Aisles too long without a break require extra time for retrievals) |  |  | P/m |  |  |  |
| E81 |  |  |  |  |  | Main Aisles safely accommodate forklift |  |  | R |  |  | R |  |  |  |
| E82 |  |  |  |  |  | Access for delivery of boxes to floors on skids |  |  | R/P (if not available will increase staff cost/time for multiple handling of shipments) |  |  | R Equipment at site |  |  |  |
| E7b |  |  |  |  |  | Floor Weight Capacity (function of ceiling height, weight load supportable, type of shelving proposed and estimated weight of records) |  |  |  |  |  |  |  |  |  |
| F |  |  |  |  |  | EQUIPMENT - This impacts on the level service which can be provided and the ease or difficulty of managing the site |  |  | Consider equipment which will come with site such as shelving and which will need to be provided or adjusted |  |  | Note what can be scheduled on workplan for change to improve efficiency/safety over time - especially before lease is signed or renewed. |  |  |  |
| F1a |  |  |  |  |  | Computers for Staff |  |  | R |  |  | m (if online inventory available) |  |  |  |
| F1a1 |  |  |  |  |  | LAN Local Area Network connecting all users at main site. |  |  | R |  |  | m (if online inventory available) |  |  |  |
| F1a2 |  |  |  |  |  | Dedicated Data Line for connection (via LAN?) to/from Headquarters for EMAIL etc |  |  | R |  |  | m (if online inventory available) |  |  |  |
| F1a3 |  |  |  |  |  | Computer printers for Staff |  |  | R |  |  | m (if online inventory available) |  |  |  |
| f1b |  |  |  |  |  | Computers for researchers, public |  |  | P |  |  | n |  |  |  |
| f1b1 |  |  |  |  |  | Any Special Data Access lines used by researchers, public for links with local or international research network |  |  | P [billing to be decided and advance notification provided] |  |  | n |  |  |  |
| F1b3 |  |  |  |  |  | Computer Printers for researchers, public |  |  | P |  |  | n |  |  |  |
| F1c |  |  |  |  |  | Photocopy machines for staff |  |  | R |  |  | m (depending on number of emergency requests for a few papers. If staff will make special trip it could be less expensive to sometimes copy needed papers on site and save expense of returning complete boxes later) |  |  |  |
| F1d |  |  |  |  |  | Photocopy machines or service for researchers, public |  |  | R [Billing pay per page, receipt, cash or check] |  |  | n |  |  |  |
| F1e |  |  |  |  |  | Phones for staff (Also Administration and Security issue - sensitive for Warehouse) |  |  | R |  |  | R (for security reasons there should be an extension within 50 meters in warehouse)and regular phone contact with central location especially if someone working alone |  |  |  |
| F1f |  |  |  |  |  | Pay Phones for Public |  |  | R |  |  | n |  |  |  |
| F1g |  |  |  |  |  | Facsimile |  |  | R |  |  | n (see note on photocopy) |  |  |  |
| F2a |  |  |  |  |  | Microfiche Reader and Reader Printer [e.g. for Official Records Series (Documents)] |  |  | R [Researcher fee schedule of paper copies or film duplicates] |  |  | n |  |  |  |
| F2b |  |  |  |  |  | Microfilm Reader and Reader printer [e.g.personnel records] |  |  | R [Researcher copy fee schedule] |  |  | n |  |  |  |
| F2c |  |  |  |  |  | CD ROM reader |  |  | R |  |  | n |  |  |  |
| F3a |  |  |  |  |  | Standard Record Containers/ Boxes (Collapsible with top 12" h, 11" wide, 18" length) |  |  | R May be changed to 10"w?, 12"h? x 15 length |  |  | R |  |  |  |
| F3b |  |  |  |  |  | Containers/ Acid Free Boxes (With tops "Hallinger Boxes" 10"ht, 5"w, 12" & 15" length) |  |  | P for some special records |  |  | n |  |  |  |
| F3d |  |  |  |  |  | Containers and Tubes for Large Oversize material [official gifts from countries or heads of State) |  |  | R |  |  | m (some short term records) |  |  |  |
| F4a |  |  |  |  |  | Desks, Workstations, and related Equipment for Offices |  |  | R |  |  | m (one office) |  |  |  |
| F4c |  |  |  |  |  | Bookcases and lateral file cabinets for Offices (15" between levels to store binders, file boxes or folders) |  |  | R |  |  | m (one office) |  |  |  |
| F4d |  |  |  |  |  | Tables, work surfaces, special sorting bins or related equipment for sorting and checking records before accessioned |  |  | R |  |  | m |  |  |  |
| F4e |  |  |  |  |  | Chairs |  |  | R |  |  | m (one office) |  |  |  |
| F4f |  |  |  |  |  | Small equipment for offices |  |  | R |  |  | m (one office) |  |  |  |
| F5a |  |  |  |  |  | Locking Lateral file cabinets to Store binders and file folders related to Historical research or retrieval requests and records of accessions and disposals. |  |  |  |  |  |  |  |  |  |
| F5b |  |  |  |  |  | Cabinets to store supplies |  |  | R |  |  | m |  |  |  |
| F6 |  |  |  | Shelving Units to store Standard cartons for records (Standard Unit = 6 levels, 72 ht, 15" Depth, 36" Width) space for 18 standard boxes on each unit or 36 Small "acid free" boxes] |  |  |  | R Standard size shelving may move toward 72" high, 15"deep, 42" wide |  |  |  |  | R Note standards in document RAM94-320/A01 |  |  |
| F6a |  |  |  | Movable units (depends on floor usage capacity [ht/weight] see also para E7) |  |  |  | P |  |  |  |  | P |  |  |
| F6b |  |  |  | Shelving Units to Store oversize Records & Gifts |  |  |  | R - without proper inventory and equipment to store these items many will self destruct from exposure. |  |  |  |  | m - for items which can not afford to be protected, after (10 years?) offer back to the government or organization donor to display in their National Museum or Headquarters |  |  |
| F7 |  |  |  | Hand Trucks, Carts |  |  |  | R |  |  |  |  | R |  |  |
| F7b |  |  |  | Fork Lift at each facility |  |  |  | R |  |  |  |  | R |  |  |
| F7c |  |  |  |  |  | Skids for records for disposal |  |  | R |  |  | R |  |  |  |
| F8 |  |  |  |  |  | Portable Industrial Fans & heaters for Warehouse |  |  | R (Should Only be necessary in emergency for archives) |  |  | R |  |  |  |
| F8a |  |  |  |  |  | Shredder - industrial size with safety equipment. In past due to personnel required and other cost/safety concerns - decided not to obtain shredder |  |  | m? |  |  | ? |  |  |  |
| Fcc |  |  |  |  |  | Containers for Flat Oversize material (e.g. Maps and Charts) |  |  | R |  |  | m (required for some records) |  |  |  |
| G |  |  |  |  |  | ADMINISTRATION (Staff, Services & supplies) |  |  |  |  |  |  |  |  |  |
| G2 |  |  |  |  |  | Staff managing facility and preparing reports on usage (see also under research area) |  |  | R |  |  | m [can be completed from Hqs site with occasional on site visit] |  |  |  |
| G2a |  |  |  |  |  | Staff for receiving/ accessioning records, marking cartons and placing on correct shelves |  |  | R |  |  | R (cartons for NCRSA should be marked for storage location before they are transferred from main site - on site only placement is required) |  |  |  |
| G2b |  |  |  |  |  | Staff for preparing records for disposal |  |  | R |  |  | R |  |  |  |
| G2c |  |  |  |  |  | Staff for physically retrieving and returning records from/to storage areas |  |  | R (can retrieve individual folder) |  |  | R [retrieve box] |  |  |  |
| G2d |  |  |  |  |  | Staff for coordination with originating offices (negotiate retention/destruction schedules and regular transfer of records) |  |  |  |  |  |  |  |  |  |
| G2e |  |  |  |  |  | Staff for training Divisional staff, researchers or public on use of archives |  |  | R |  |  | n |  |  |  |
| G2f |  |  |  |  |  | Staff for training Divisional users in correct record handling procedures for current records (& support preparation for future accessions) |  |  | R |  |  | n |  |  |  |
| G3a |  |  |  |  |  | Staff for Security on Site (see also para. A = Security) |  |  | R |  |  | n |  |  |  |
| G3b |  |  |  |  |  | Receptionist |  |  | R |  |  | n |  |  |  |
| G3c |  |  |  |  |  | Research related Staff (see also para. C = Research) |  |  | R |  |  | n (provided by requesting office, Records Management or Archives staff at Hqs) |  |  |  |
| G4 |  |  |  |  |  | Inventory of Space and box locations available on line |  |  | R |  |  | P (if all space is assigned before accepted) |  |  |  |
| G5 |  |  |  |  |  | Office Cleaning Service |  |  | R |  |  | m |  |  |  |
| G5a |  |  |  |  |  | Garbage Removal Service |  |  | R |  |  | R |  |  |  |
| G5c |  |  |  |  |  | Toilet - Cleaning Service |  |  | R |  |  | R [Required periodically] |  |  |  |
| G6a |  |  |  |  |  | Office Supplies |  |  | R |  |  | m |  |  |  |
| G6s |  |  |  |  |  | Toilet supplies |  |  | R |  |  | R [Should be supply cabinet - periodically resupplied] (Staff sometimes had to purchase at LIC warehouse) |  |  |  |
| H |  |  |  |  |  | Legal Issues (tests and Certifications - see also Physical Plant para I) |  |  | See also Para J for additional concerns when considering emergency storage of UN related records in NON-UN controlled facility |  |  |  |  |  |  |
| H1 |  |  |  |  |  | Lease Agreement |  |  | R |  |  | R |  |  |  |
| H1a |  |  |  |  |  | Length of Lease Agreement |  |  | R |  |  | R |  |  |  |
| H1a1 |  |  |  |  |  | Option for Renewal (length) Option to Cancel. Penalty or advantages spelled out. |  |  | R - length of notice required can impact on relocation costs |  |  | R |  |  |  |
| H1b |  |  |  |  |  | "Privileges and Immunities" of UN and records to be respected. |  |  | R |  |  | R |  |  |  |
| H1b |  |  |  |  |  | Certificate of Occupancy |  |  | R |  |  | R |  |  |  |
| H1c |  |  |  |  |  | Title to Building with Agent/Landlord- copy received |  |  | R |  |  | R |  |  |  |
| H2a |  |  |  |  |  | Tests for Air quality on File within past 12 months. |  |  | R |  |  | m |  |  |  |
| H2b |  |  |  |  |  | Tests for structural soundness (completed within 12 months before occupancy and before any major renovation or addition, eg compact shelving) |  |  | R |  |  | m |  |  |  |
| H2b1 |  |  |  |  |  | Tests for Roofing soundness (visit during/after heavy rain -be aware of air conditioning units on the roof and drains for runoff in case of breakdowns or leaks) |  |  | R |  |  | m |  |  |  |
| H2c |  |  |  |  |  | Fire Protection Certified by local agency responsible for service within past 12 months |  |  | R Note also Security Para A7a Fire Protection |  |  | R |  |  |  |
| H2d |  |  |  |  |  | Test for Hazardous Building Materials within past 3 years. |  |  | R Note also Environmental Concerns para B3: Asbestos etc. |  |  | R |  |  |  |
| I |  |  |  |  |  | Physical Plant |  |  |  |  |  |  |  |  |  |
| I1a |  |  |  |  |  | Electrical Wiring sufficient for all equipment and lighting identified in other paragraphs. |  |  | R |  |  | R |  |  |  |
| I1b |  |  |  |  |  | Reliable Power Source for all probable electrical needs identified in other paragraphs. |  |  | R |  |  | R |  |  |  |
| I1c |  |  |  |  |  | Wiring for Computer connections |  |  | R |  |  | m |  |  |  |
| I2 |  |  |  |  |  | No Exposed Pipes or Electrical Connections |  |  | R |  |  | m |  |  |  |
| I3a |  |  |  |  |  | Air Cooling Equipment Tested Reliable |  |  | R |  |  | m |  |  |  |
| I3b |  |  |  |  |  | Heating Equipment Tested Reliable |  |  | R |  |  | m |  |  |  |
| I4a |  |  |  |  |  | Aisles and Units Clearly Marked and Labeled |  |  | R |  |  | R |  |  |  |
| I4b |  |  |  |  |  | Exits sufficient for emergencies - Clearly marked |  |  | R |  |  | R |  |  |  |
| I5 |  |  |  |  |  | Toilets (Sufficient size and number for amount of people as identified in other paragraphs) |  |  | R |  |  | R |  |  |  |
| I5a |  |  |  |  |  | Showers for staff. Especially those who must regularly move or handel boxes from storage areas. |  |  | P |  |  | P |  |  |  |
| I6 |  |  |  | Elevators tested for weight, size and number to be sufficient for people and equipment identified in other paragraphs. |  |  |  | R |  |  |  |  | R [if more than one level on site] |  |  |

| J |  |  | Commercial Facilities possible Use for Temporary Records | Additional Concerns for possible use due to emergency or non availability of suitable UN controlled storage areas. |  |
| --- | --- | --- | --- | --- | --- |
| J1 |  |  | Meet Standard for the appropriate storage area AVRSA or NCRSA & (signed agreement to maintain the standard) | R | R |
| J1a |  |  | Decision by UN Legal Office and other authorities that use of a Commercial facility will not compromise the inviolability of specific types of records and/or protection of those records according to international practice or convention. | R | R |
| J1b |  |  | Clearance of UN legal office of agreement to use a specific commercial facility for type of records proposed. | R | R |
| J1c |  |  | Host Country Agreement - by Government to respect Privileges and Immunity of UN and Records | R | R |
| J1d |  |  | Agreement of Commercial facility and Host country to fully respect the records as property of the UN or Agency not to be violated. | R | R |
| J2 |  |  | Access 24 hours a day in Emergency (or mutually agreed times and duration of prior notification required with estimated cost) to all Authorized UN or Agency RAM or Security personnel. | R - Letter of Understanding should state all of the agreements noted below and under what conditions some temporary records might be handled or disposed after appraisal is completed. | R |
| J2a |  |  | Without prior notification [during regular scheduled business hours], agreement for Authorized UN or Agency RAM or Security personnel to be granted access to all areas in which UN Records are processed or transported within Agency and to inspect all areas to test the compliance to agreed standards. | R | R |
| J2b |  |  | All personnel handling UN Records are "bonded" or certified to be competent by Commercial facility. | R | R |
| J2b1 |  |  | Lists of personnel available for inspection by UN Agency RAM or Security Staff. Agreement by Commercial facility to provide access to employees with instruction to answer related questions concerning record handling procedures and equipment. | R | P |
| J2b2 |  |  | No non authorized personnel [as set out in agreement] will be allowed to inspect UN or Agency records. This includes list of records, contents or transaction/retrieval logs. [signed agreement] | R | R |
| J2d |  |  | Agreement to Prohibit other than employees of Commercial facility to Transport records or otherwise handle containers. | R Destruction can only take place in presence of UN ARCHIVE or UN Agency RAM staff as set out in Agreement | R |
| J3e |  |  | Agreement for employees not to inspect contents of containers without specific permission of authorized UN or Agency RAM Official. Each inspection to be logged with authorizing official name and office. | R | R |
| J3f |  |  | On line database of all commercial storage of UN or Agency records including location of container. | R | R |
| J3f1 |  |  | Commercial Facility Database of Location easily uploaded to UN or agency RAM Computer to link to Contents of Containers | P | P |
| J3f2 |  |  | Database includes details of Transaction dates etc. | P | P |
| J3f3 |  |  | Billing easily traced through data base linked to shipment transfers, specific retrievals or other actions agreed by UN or Agency RAM. | P | P |
| J3f4 |  |  | Records stored by Commercial facility easily requested by user departments with approval of RAM unit. | R | R |
| J3f5 |  |  | Descriptions or Records [to folder level] held by Commercial facility appear "on screen" to users as easily as current records in system. | P | P |
| J4 |  |  | Delivery Times - 24 Hour Maximum when required. Available in 3 hours on exceptional basis and with special charge if agreed. | R | R |
| J6 |  |  |  |  |  |